NVIDIA Jetson TX2 Configuration and Implementation

Get the host PC (Ubuntu 18.04). Refer to the Jetson TX2 developer kit, read through it and then click on the link to download and install the SDK manager.

Download for Ubuntu (.deb x86_64), then run the following command in the terminal: sudo apt install ./sdkmanager_2.4.1-13536_amd64.deb

(if it fails to work, finish the current task (APT in our example) and acquire the lock. Then reboot and proceed)

Launch sdkmanager in terminal and then login.

(ensure that the nvidia server time matches the system time for smooth installation)

Ensure that the port connections between the monitor, host pc, and the board are correct. Also ensure that the mouse and keyboard are connected to the board. And perform the steps to enable force USB recovery mode.

Once done,

Step 1: Select the correct hardware device/board (Jetson TX2 P2597) on the Host PC and make sure it is recognised. Proceed to step 2.

Step 2: check the accept the policy box and start downloading.

Step 3: Give the username and password and as mentioned in the Quick Start Guide, select the automatic mode, click on Flash and wait for downloads & installs to complete.

HDMI Monitor boot up:

Accept T&C, ensure correct language, keyboard and timezone settings while setting up. Then, connect to a wireless network and proceed to account creation - pick a username and password.

In our case, neither the ethernet nor the micro-usb connection worked, so we downloaded packages via mobile wifi using the following commands:

ping -c 3 8.8.8.8 - (confirms if connections are proper for downloading)

sudo apt update

sudo apt install nvidia-jetpack

Let it run, then verify the version using: nvcc --version. It said bash: nvcc command not found.

Check if CUDA (Compute Unified Device Architecture) is properly installed using: ls /usr/local/ | grep cuda

This returned the versions of CUDA as

cuda

cuda-10

cuda-10.2

To fix the path and get nvcc - -version, we ran the following:

echo ‘export PATH=/usr/local/cuda/bin:$PATH’ >> ~/.bashrc

echo ‘export LD_LIBRARY_Path=/usr/local/cuda/lib64:$LD_LIBRARY_PATH’ >> ~/.bashrc

source ~/.bashrc

nvcc --version

After this, we got the expected output, getting the release versions and cuda versions.

Run the following commands:

(dpkg is the package manager used by Debian-based Linux distributions such as Ubuntu. The -l (lowercase L) option means list installed packages. grep cudnn returns only lines containing the word cudnn.)

dpkg -l | grep cudnn

In the output each line represents a package installed on the system.

Similarly, check for tensorrt (base to run Tensorflow) using: dpkg -l | grep tensorrt

Also, verify the version of python3 (python3 --version)

Then, proceed to install pip with: sudo apt install python3-pip

Upgrade it using: pip3 install --upgrade pip

Use pip3 list to make sure that the necessary modules are installed (like numpy)

Verify the version of jetpack installed using: dpkg -l | grep nvidia-jetpack

Keras was not installed, so we proceeded to install it with

Running the commands at once as a pipeline in a single line of command didn’t work, so we had to individually run to install different packages.

General command line code was:

sudo pip3 install -U --no-deps --default-timeout=200 {package_to_install}

Packages:

- numpy (1.19.4)

- future (0.18.2)

- mock (3.0.5)

- keras_preprocessing (1.1.2)

- keras_applications (1.0.8)

- gast (0.4.0)

- protobuf (3.19.6)

- pybind11

- cython (0.29.37)

- pkgconfig

Eg: sudo pip3 install -U –no-deps --default-timeout=200 mock==3.0.5

Once all the necessary packages were installed, we installed ran the following command:

sudo env H5PY_SETUP_REQUIRES=0 pip3 install -U h5py=3.1.0

which was needed for tf and keras.

Then we wrote the following code that downloads Tensorflow:

sudo pip3 install --pre --extra-index-ur https://developer.download.nvidia.com/compute/redist/jp/v46 ‘tensorflow<2.10’

To check if imports were proper:

python3 -c “import tensorflow as tf; print(tf.__version__); from tensorflow import keras; print(‘keras OK’)”

In our case, we didn’t specify the versions of cython and protobuf, which caused the following errors:

ERROR: Could not build wheels for h5py, which is required to install pyproject.toml-based projects

This was fixed for cython by running sudo pip3 install “cython<3”

Similarly since protobuf wasnt in correct version, import tensorflow didnt work properly. We fixed that by:

sudo pip3 install --force-reinstall --no-cache-dir --no-deps protobuf==3.19.6

Finally, to test if the GPU is configured correctly, we ran:

python3 -c “import tensorflow as tf; print(‘GPU available:’, tf.config.list_physical_devices(‘GPU))”